Lecture 1

- Routing

  - Paths

- Data Links – physical layer

  - Between 2 nodes (error detection, etc.)

- Narrow physical layer restriction:

  - Need to send data in small bits

- DNS – directory service -> name to ip address

- Network header

  - Routing info (all intermediary routers)

- Data link header

  - Only info about next router

- Physical layer

  - No header, splits DPDU (?) (frame) into bits

- At destination node, data link header is taken off, then the network header. Then, it sees final destination is this router and proceeds to take off other headers

- Data links – fiber, wireless, cable, Bluetooth, etc.

- TCP vs. UDP transport layer

  - UDP – no resending/retransmission

  - TCP – more reliable

- TCP abstraction

  - Sender and receiver have 2 connected queues and data reliably is sent from sender to receiver queue. No packets.

  - Uses data link to send packets between 2 routers, which sends bits at physical layer

Lecture 2

- TCP

  - Queue abstraction

- IP layer

  - IP header places source and destination

  - If IP addresses match, then we are at the right destination

  - Else, figure out the next router

- Data link

  - From mac address x to y

- Mac address

  - Unique 48 bit addresses uniquely identify everyone on a link

  - E.g., x->y z->l, y,z are mac addresses for the same router

- IP address

  - Temporarily assigned to device

- Physical

  - Bits

- RPC (remote procedure calls) vs TCP (Queue abstraction)

- Socket = queue

- Transmission

  - Bandwidth

    - Range of frequencies a particular signal is created from (infinite sum of which sine wave freqs)

  - Strict layering

    - Only read headers at current layer (e.g., IP only reads IP header)

    - Information can only be passed across interfaces

  - Physical layer

    - Frequency shifting vs amplitude shifting (baseband)

    - Bits to coded bits to add transitions

    - Send bits to modem to convert to energy

  - Channel

    - Physical medium to transmit from sender to receiver

    - A channel does not change frequency of sine waves (only maybe the amplitude and phase)

  - Any periodic function can be written as an infinite sum of sin waves with diff freqs

  - More harmonics/bandwidth -> better bit recovery

  - Harmonic = multiple of the fundamental frequency

  - Channels are sluggish (take time to respond to a change in the signal)

  - Channels have noise, resulting in a change in amplitude

  - Sample at middle points

  - Two limits

    - Bandwidth (Nyquist rate) – limit on symbols (signals)

      - Input: square wave, output: falling sine wave

        - When can be send the second signal such that it does not interfere with previous signal?

          - Bandwidth of input signal is 1/T

          - You can theoretically send signals every T/2 seconds (Send at frequency: 2 * bandwidth = 2/T)

            - Max rate of sending symbols, not bits (baud rate)

      - Why not send multiple bits per symbol (6v – 11, 4v – 10, …)

        - Noise

    - Noise (Shannon) – limit on bits

      - Maximum number of noise to tolerate to distinguish 6V and 4V is 1V, which limits the number of bits per symbol

      - Shannon limit applies to any channel (wire, wireless, etc.)

      - S – max signal

      - N – noise

      - Signals must be spaced 2*N

      - B – bandwidth

      - Shannon limit is B * log (1 + S/2N) bits per second

- Channel needs to pass a range of frequencies in a range from highest to 0Hz

  - 010101010101 – highest

  - 000000000000 – 0 Hz

  - Non-periodic bit patterns

Lecture 4

- Physical layer

  - Coding sublayer

    - Add transition bits

    - Initial training bits used to sync clock at the beginning

      - Physical clocks drift over time -> we need transition bits to keep them in sync

    - Example

      - Transition bits

        - Start detected by rising signal (edge detector)

        - Sample every 1 bit after that

      - Initial transition bit after each character being transmitted

    - Manchester

      - Sends double the number of bits (if Nyquist limit is B, we send B/2 amount of useful data)

      - 1->0 : 1

      - 0->1 : 0

      - Do not look at the signal for about 20-30%.

        - E.g., to send data 1111111

        - We need, 10-10-10-10-10-10-10

        - We must ignore the 01 transitions

        - No need to look at a specific point (middle of a peak), instead we watch for a transition from 0 to 1 or 1 to 0

      - The data 0101011 indicates start of the bit pattern (preamble phase)

      - Looking for rising and falling edges is dangerous due to noise

    - Phase Locked Loops (?)

    - Eye pattern (?)

      - Superimpose 2 signals together

    - Multiplexing (n times more bandwidth by sending more data at once)

      - Time

      - Frequency

      - Wavelength (visible light)

        - Send red, blue, etc. signals at the same time at the sender end

      - CDMA

        - Different frequencies at different times

    - Async vs sync

      - Async: Sender and receiver don’t have the same clock. Need start and stop bits and an accepted number of data bits (e.g., 1 byte)

      - Sync: Both the sender and receiver have synchronized clocks. Need transitions to ensure clocks do not drift.

  - Decoding sublayer

    - Clock recovery

  - Media transmission sublayer

    - Convert digital bits to energy (electricity or light)

  - Signal transmission sublayer

    - What the channel does to the signal (Nyquist and Shannon limits)

Lecture 5

- www.google.com -> DNS -> IP address

  - IP -> 32 bits

- HTTP atop TCP

  - HTTP header

- TCP adds sequence numbers to order data when received (after retransmission)

- Routing

  - Each router knows the next hop (forwarding table) and passes that to data link

- Data link

  - MAC in header

- RS-232

  - Interface between CPU and modem connected to twisted pair

- Cable

  - Twisted pair (last mile)

    - Signal is difference between both pairs -> signal gets cancelled out

    - Used to be low battery and low throughput and long distance

      - Now higher throughput less distance (cat 5/6 twisted pair)

    - Cable = coaxial

  - Cable is good at sending data to you

  - Coaxial

    - Used bw buildings

  - Twisted pair

    - Local

- Fibre

  - A lot of bandwidth

  - Each fibre cable is unidirectional

  - Total internal reflection – no refraction into the fibre cable

  - Multimode fibre

    - One part reflects (long path)

    - One part doesn’t (short path)

    - Limits bandwidth

  - Single mode -- Thin fibre

    - Wave will not be spread into parts... just one thin palse

    - Higher bandwidth because there is no dispersion (wave is not spread out)

  - Use monochromatic lasers so that the light waves do not interfere

- Wireless

  - Spectrum

    - Radio waves

      - Omnidirectional – goes in all direction

      - Goes through obstacles

    - Satiates, microwaves

      - Directional

      - Absorbed by obstacles

      - Signal to satellite from dish will be unidirectional

      - The signal back from satellite to its destination will have a larger direction range

      - Geosynchronous

        - Satellite should rotate around earth at same speed earth rotates itself

          - Antenna only needs to point in one directions

        - Satellite needs to be far to achieve geosynchrony

          - Huge latency

          - Not as much bandwidth (cannot keep too many satellites in the sky)

      - Right of way

        - Satellites are not on the ground (bypass wires, …)

      - Low orbiting satellites

        - More latency

        - Cover earth with low orbiting satellites… handoff connection to closest satellite as one rotates away from dish

        - (satellites are not geosynchronous since they are too close)

        - Disadvantage: You need a lot of satellites that span around earth

- FMD = freq division multiplex

- TDM = time

- WDM = wavelength

- Interconnection

  - Router

    - More latency

    - Can redirect if a link goes down

    - Check for errors

  - Repeater (“hubs”)

    - Less latency

    - Cheaper

    - Physical layer

    - “Blindly” repeat bits

    - Fibre stays at the repeater level

Data link

- Hop by hop should be an optimization (so that routers retransmit before TCP times out… not worth it if links are reliable)

- End-to-end ack is required to guarantee successful transmission

- Why framing

  - Headers

    - Individual bits cannot have headers (which would include destination, source, etc.)

  - Multiplexing

    - 2 senders and 2 receivers share a link

      - Each sender sends 1 frame at a time

    - Need frames with headers to multiplex (need discrete units to multiplex)

  - Error recovery

    - Easier to retransmit frames rether than the entire data stream

  - Frame boundaries

- Sol 1 : High Level Data-Link Control (HDLC)

  - Flag: 01111110

    - Start and stop delimeter

  - Bit stuffing

    - If data has pattern 11111, then add a 0

    - So, 111110 -> 11111

    - So, 1111100 -> 111110

    - Stuff an extra 0 after 5 ones

- Sol 2 : Ethernet

  - Inter-frame gap

    - Period of silence (0 V)

    - For 96 bit times

  - Period of preamble (56 bits)

    - 10101010…

    - For receiver to synchronize clock with sender

  - Start of frame delimeter

    - 10101011

  - Data

    - Includes header (dest, src, …)

  - Inter-frame gap

    - Period of silence (0V)

- Too small frames

  - Error rate

- Too large frames

  - Higher chance of error and need to recover a larger number of bits

- Usual amount: 64 – 1500 bytes

- If flag is 01111110, should add 0 after 5 1’s, not after 011111

  - First case: 0111111111110 -> 011111011111010 – transmit – decode -> 011111111110 (good)

  - Second case: 01111101111110 -> deflagger will see 01111110 as a EOF delimeter -> data after this pattern is lost

- If flag is 01010101, cannot stuff 1 after 010101

  - Data = 0101

  - Transmitted = 01010101 0101 01010101

        - -------------

        - Receiver sees this as a end flag -> data lost

- Sol 3 : Change the physical API

  - E.g., 4-5 encoding gives 0000 to 1111 data values, but there are many other bit patterns not used. Use those as control characters (SOF and EOF)

  - Add a third symbol at the physical layer (0 -> 0V, 1 -> 1V, frame boundary -> 5V?)

- Errors

  - Random error

    - 0 or 1 corrupted

    - Noise corrupts 1 bit

  - Burst error of length n

    - Bit 1 and n are corrupted. Bits in between may be corrupted

  - H(M) is 32 bits -> 2^32 possibilities -> 1/2^32 chance of getting the same hash for M and M’ such that M != M’ -> undetected error

    - Assuming errors and hash are random

  - Error correction is more expensive, not worth it when retransmission is few ms

  - xxxxxxx -> xxxxxxxppp -> ppp -> needed addition to make xxxxxxxppp divisible by 7 -> receiver will ensure xxxxxxxppp is divisible by 7 -> extract xxxxxxx as data bits (we add 3 bits because we add up to 6 to make num divisible by 7 -> 110 is 3 bits.

  - CRC uses mod 2 arithmetic

    - Addition = subtraction = xor

      - 10001 + 11100 = 01101

    - Multiply

      - Shift + XOR

      - 10111 * 11 = 11001 (shifting just as normal mult -> use XOR when adding the terms)

    - Division

      - M = 1001, G = 101 => r = 3 (num bits of G)  [If first bit of G is 1, then the remainder will be r-1 bits)

      - M’ = 100100 (add r-1 = 2 bits to M)

      - 1011

      - _______

      - 101 | 100100

      - 101

      - 0011

      - 000

      - 110

      - 101

      - 0110

      - 101

      - 011

      - Remainder t = 11 (CRC).    [you no longer take G-t as CRC (only t)]

      - Final message = 100111 (which is mod 2 divisible by G=101

Each codeword has a set of valid codewords.

The idea is to maximize hamming distance between two adjacent valid codewords.

Parity has a hamming distance of 2

0 -> 000

1 -> 111

Hamming distance is 3

Hamming distance – 1 == resilience to error

Hamming distance of d:

C1 -------- d ---------- C2

|

|

|

d

|

|

|

C3

Any error of d – 1 bits on a valid codeword will result in an invalid codeword, so it will be dropped. => All d – 1 bit errors will be detected

Hamming distance of CRC is 4 (all 1-bit and 2-bit errors are detected… all odd bit errors are detected (3), but 4 bit errors are not detected)

Error correction: If hamming distance is 2d + 1, we can correct all d bit errors by going to the closest valid codeword. Error of > d bits will result in an incorrect correction