Lecture 1

- Routing

  - Paths

- Data Links – physical layer

  - Between 2 nodes (error detection, etc.)

- Narrow physical layer restriction:

  - Need to send data in small bits

- DNS – directory service -> name to ip address

- Network header

  - Routing info (all intermediary routers)

- Data link header

  - Only info about next router

- Physical layer

  - No header, splits DPDU (?) (frame) into bits

- At destination node, data link header is taken off, then the network header. Then, it sees final destination is this router and proceeds to take off other headers

- Data links – fiber, wireless, cable, Bluetooth, etc.

- TCP vs. UDP transport layer

  - UDP – no resending/retransmission

  - TCP – more reliable

- TCP abstraction

  - Sender and receiver have 2 connected queues and data reliably is sent from sender to receiver queue. No packets.

  - Uses data link to send packets between 2 routers, which sends bits at physical layer

Lecture 2

- TCP

  - Queue abstraction

- IP layer

  - IP header places source and destination

  - If IP addresses match, then we are at the right destination

  - Else, figure out the next router

- Data link

  - From mac address x to y

- Mac address

  - Unique 48 bit addresses uniquely identify everyone on a link

  - E.g., x->y z->l, y,z are mac addresses for the same router

- IP address

  - Temporarily assigned to device

- Physical

  - Bits

- RPC (remote procedure calls) vs TCP (Queue abstraction)

- Socket = queue

- Transmission

  - Bandwidth

    - Range of frequencies a particular signal is created from (infinite sum of which sine wave freqs)

  - Strict layering

    - Only read headers at current layer (e.g., IP only reads IP header)

    - Information can only be passed across interfaces

  - Physical layer

    - Frequency shifting vs amplitude shifting (baseband)

    - Bits to coded bits to add transitions

    - Send bits to modem to convert to energy

  - Channel

    - Physical medium to transmit from sender to receiver

    - A channel does not change frequency of sine waves (only maybe the amplitude and phase)

  - Any periodic function can be written as an infinite sum of sin waves with diff freqs

  - More harmonics/bandwidth -> better bit recovery

  - Harmonic = multiple of the fundamental frequency

  - Channels are sluggish (take time to respond to a change in the signal)

  - Channels have noise, resulting in a change in amplitude

  - Sample at middle points

  - Two limits

    - Bandwidth (Nyquist rate) – limit on symbols (signals)

      - Input: square wave, output: falling sine wave

        - When can be send the second signal such that it does not interfere with previous signal?

          - Bandwidth of input signal is 1/T

          - You can theoretically send signals every T/2 seconds (Send at frequency: 2 * bandwidth = 2/T)

            - Max rate of sending symbols, not bits (baud rate)

      - Why not send multiple bits per symbol (6v – 11, 4v – 10, …)

        - Noise

    - Noise (Shannon) – limit on bits

      - Maximum number of noise to tolerate to distinguish 6V and 4V is 1V, which limits the number of bits per symbol

      - Shannon limit applies to any channel (wire, wireless, etc.)

      - S – max signal

      - N – noise

      - Signals must be spaced 2*N

      - B – bandwidth

      - Shannon limit is B * log (1 + S/2N) bits per second

- Channel needs to pass a range of frequencies in a range from highest to 0Hz

  - 010101010101 – highest

  - 000000000000 – 0 Hz

  - Non-periodic bit patterns

Lecture 4

- Physical layer

  - Coding sublayer

    - Add transition bits

    - Initial training bits used to sync clock at the beginning

      - Physical clocks drift over time -> we need transition bits to keep them in sync

    - Example

      - Transition bits

        - Start detected by rising signal (edge detector)

        - Sample every 1 bit after that

      - Initial transition bit after each character being transmitted

    - Manchester

      - Sends double the number of bits (if Nyquist limit is B, we send B/2 amount of useful data)

      - 1->0 : 1

      - 0->1 : 0

      - Do not look at the signal for about 20-30%.

        - E.g., to send data 1111111

        - We need, 10-10-10-10-10-10-10

        - We must ignore the 01 transitions

        - No need to look at a specific point (middle of a peak), instead we watch for a transition from 0 to 1 or 1 to 0

      - The data 0101011 indicates start of the bit pattern (preamble phase)

      - Looking for rising and falling edges is dangerous due to noise

    - Phase Locked Loops (?)

    - Eye pattern (?)

      - Superimpose 2 signals together

    - Multiplexing (n times more bandwidth by sending more data at once)

      - Time

      - Frequency

      - Wavelength (visible light)

        - Send red, blue, etc. signals at the same time at the sender end

      - CDMA (?)

        - Different frequencies at different times

    - Async vs sync (?)

  - Decoding sublayer

    - Clock recovery

  - Media transmission sublayer

    - Convert digital bits to energy (electricity or light)

  - Signal transmission sublayer

    - What the channel does to the signal (Nyquist and Shannon limits)