Lecture 1

- Routing

  - Paths

- Data Links – physical layer

  - Between 2 nodes (error detection, etc.)

- Narrow physical layer restriction:

  - Need to send data in small bits

- DNS – directory service -> name to ip address

- Network header

  - Routing info (all intermediary routers)

- Data link header

  - Only info about next router

- Physical layer

  - No header, splits DPDU (?) (frame) into bits

- At destination node, data link header is taken off, then the network header. Then, it sees final destination is this router and proceeds to take off other headers

- Data links – fiber, wireless, cable, Bluetooth, etc.

- TCP vs. UDP transport layer

  - UDP – no resending/retransmission

  - TCP – more reliable

- TCP abstraction

  - Sender and receiver have 2 connected queues and data reliably is sent from sender to receiver queue. No packets.

  - Uses data link to send packets between 2 routers, which sends bits at physical layer

Lecture 2

- TCP

  - Queue abstraction

- IP layer

  - IP header places source and destination

  - If IP addresses match, then we are at the right destination

  - Else, figure out the next router

- Data link

  - From mac address x to y

- Mac address

  - Unique 48 bit addresses uniquely identify everyone on a link

  - E.g., x->y z->l, y,z are mac addresses for the same router

- IP address

  - Temporarily assigned to device

- Physical

  - Bits

- RPC (remote procedure calls) vs TCP (Queue abstraction)

- Socket = queue

- Transmission

  - Bandwidth

    - Range of frequencies a particular signal is created from (infinite sum of which sine wave freqs)

  - Strict layering

    - Only read headers at current layer (e.g., IP only reads IP header)

    - Information can only be passed across interfaces

  - Physical layer

    - Frequency shifting vs amplitude shifting (baseband)

    - Bits to coded bits to add transitions

    - Send bits to modem to convert to energy

  - Channel

    - Physical medium to transmit from sender to receiver

    - A channel does not change frequency of sine waves (only maybe the amplitude and phase)

  - Any periodic function can be written as an infinite sum of sin waves with diff freqs

  - More harmonics/bandwidth -> better bit recovery

  - Harmonic = multiple of the fundamental frequency

  - Channels are sluggish (take time to respond to a change in the signal)

  - Channels have noise, resulting in a change in amplitude

  - Sample at middle points

  - Two limits

    - Bandwidth (Nyquist rate) – limit on symbols (signals)

      - Input: square wave, output: falling sine wave

        - When can be send the second signal such that it does not interfere with previous signal?

          - Bandwidth of input signal is 1/T

          - You can theoretically send signals every T/2 seconds (Send at frequency: 2 * bandwidth = 2/T)

            - Max rate of sending symbols, not bits (baud rate)

      - Why not send multiple bits per symbol (6v – 11, 4v – 10, …)

        - Noise

    - Noise (Shannon) – limit on bits

      - Maximum number of noise to tolerate to distinguish 6V and 4V is 1V, which limits the number of bits per symbol

      - Shannon limit applies to any channel (wire, wireless, etc.)

      - S – max signal

      - N – noise

      - Signals must be spaced 2*N

      - B – bandwidth

      - Shannon limit is B * log (1 + S/2N) bits per second

- Channel needs to pass a range of frequencies in a range from highest to 0Hz

  - 010101010101 – highest

  - 000000000000 – 0 Hz

  - Non-periodic bit patterns

Lecture 4

- Physical layer

  - Coding sublayer

    - Add transition bits

    - Initial training bits used to sync clock at the beginning

      - Physical clocks drift over time -> we need transition bits to keep them in sync

    - Example

      - Transition bits

        - Start detected by rising signal (edge detector)

        - Sample every 1 bit after that

      - Initial transition bit after each character being transmitted

    - Manchester

      - Sends double the number of bits (if Nyquist limit is B, we send B/2 amount of useful data)

      - 1->0 : 1

      - 0->1 : 0

      - Do not look at the signal for about 20-30%.

        - E.g., to send data 1111111

        - We need, 10-10-10-10-10-10-10

        - We must ignore the 01 transitions

        - No need to look at a specific point (middle of a peak), instead we watch for a transition from 0 to 1 or 1 to 0

      - The data 0101011 indicates start of the bit pattern (preamble phase)

      - Looking for rising and falling edges is dangerous due to noise

    - Phase Locked Loops (?)

    - Eye pattern (?)

      - Superimpose 2 signals together

    - Multiplexing (n times more bandwidth by sending more data at once)

      - Time

      - Frequency

      - Wavelength (visible light)

        - Send red, blue, etc. signals at the same time at the sender end

      - CDMA

        - Different frequencies at different times

    - Async vs sync

      - Async: Sender and receiver don’t have the same clock. Need start and stop bits and an accepted number of data bits (e.g., 1 byte)

      - Sync: Both the sender and receiver have synchronized clocks. Need transitions to ensure clocks do not drift.

  - Decoding sublayer

    - Clock recovery

  - Media transmission sublayer

    - Convert digital bits to energy (electricity or light)

  - Signal transmission sublayer

    - What the channel does to the signal (Nyquist and Shannon limits)

Lecture 5

- www.google.com -> DNS -> IP address

  - IP -> 32 bits

- HTTP atop TCP

  - HTTP header

- TCP adds sequence numbers to order data when received (after retransmission)

- Routing

  - Each router knows the next hop (forwarding table) and passes that to data link

- Data link

  - MAC in header

- RS-232

  - Interface between CPU and modem connected to twisted pair

- Cable

  - Twisted pair (last mile)

    - Signal is difference between both pairs -> signal gets cancelled out

    - Used to be low battery and low throughput and long distance

      - Now higher throughput less distance (cat 5/6 twisted pair)

    - Cable = coaxial

  - Cable is good at sending data to you

  - Coaxial

    - Used bw buildings

  - Twisted pair

    - Local

- Fibre

  - A lot of bandwidth

  - Each fibre cable is unidirectional

  - Total internal reflection – no refraction into the fibre cable

  - Multimode fibre

    - One part reflects (long path)

    - One part doesn’t (short path)

    - Limits bandwidth

  - Single mode -- Thin fibre

    - Wave will not be spread into parts... just one thin palse

    - Higher bandwidth because there is no dispersion (wave is not spread out)

  - Use monochromatic lasers so that the light waves do not interfere

- Wireless

  - Spectrum

    - Radio waves

      - Omnidirectional – goes in all direction

      - Goes through obstacles

    - Satiates, microwaves

      - Directional

      - Absorbed by obstacles

      - Signal to satellite from dish will be unidirectional

      - The signal back from satellite to its destination will have a larger direction range

      - Geosynchronous

        - Satellite should rotate around earth at same speed earth rotates itself

          - Antenna only needs to point in one directions

        - Satellite needs to be far to achieve geosynchrony

          - Huge latency

          - Not as much bandwidth (cannot keep too many satellites in the sky)

      - Right of way

        - Satellites are not on the ground (bypass wires, …)

      - Low orbiting satellites

        - More latency

        - Cover earth with low orbiting satellites… handoff connection to closest satellite as one rotates away from dish

        - (satellites are not geosynchronous since they are too close)

        - Disadvantage: You need a lot of satellites that span around earth

- FMD = freq division multiplex

- TDM = time

- WDM = wavelength

- Interconnection

  - Router

    - More latency

    - Can redirect if a link goes down

    - Check for errors

  - Repeater (“hubs”)

    - Less latency

    - Cheaper

    - Physical layer

    - “Blindly” repeat bits

    - Fibre stays at the repeater level

Data link

- Hop by hop should be an optimization (so that routers retransmit before TCP times out… not worth it if links are reliable)

- End-to-end ack is required to guarantee successful transmission

- Why framing

  - Headers

    - Individual bits cannot have headers (which would include destination, source, etc.)

  - Multiplexing

    - 2 senders and 2 receivers share a link

      - Each sender sends 1 frame at a time

    - Need frames with headers to multiplex (need discrete units to multiplex)

  - Error recovery

    - Easier to retransmit frames rether than the entire data stream

  - Frame boundaries

- Sol 1 : High Level Data-Link Control (HDLC)

  - Flag: 01111110

    - Start and stop delimeter

  - Bit stuffing

    - If data has pattern 11111, then add a 0

    - So, 111110 -> 11111

    - So, 1111100 -> 111110

    - Stuff an extra 0 after 5 ones

- Sol 2 : Ethernet

  - Inter-frame gap

    - Period of silence (0 V)

    - For 96 bit times

  - Period of preamble (56 bits)

    - 10101010…

    - For receiver to synchronize clock with sender

  - Start of frame delimeter

    - 10101011

  - Data

    - Includes header (dest, src, …)

  - Inter-frame gap

    - Period of silence (0V)

- Too small frames

  - Error rate

- Too large frames

  - Higher chance of error and need to recover a larger number of bits

- Usual amount: 64 – 1500 bytes

- If flag is 01111110, should add 0 after 5 1’s, not after 011111

  - First case: 0111111111110 -> 011111011111010 – transmit – decode -> 011111111110 (good)

  - Second case: 01111101111110 -> deflagger will see 01111110 as a EOF delimeter -> data after this pattern is lost

- If flag is 01010101, cannot stuff 1 after 010101

  - Data = 0101

  - Transmitted = 01010101 0101 01010101

        - -------------

        - Receiver sees this as a end flag -> data lost

- Sol 3 : Change the physical API

  - E.g., 4-5 encoding gives 0000 to 1111 data values, but there are many other bit patterns not used. Use those as control characters (SOF and EOF)

  - Add a third symbol at the physical layer (0 -> 0V, 1 -> 1V, frame boundary -> 5V?)

- Errors

  - Random error

    - 0 or 1 corrupted

    - Noise corrupts 1 bit

  - Burst error of length n

    - Bit 1 and n are corrupted. Bits in between may be corrupted

  - H(M) is 32 bits -> 2^32 possibilities -> 1/2^32 chance of getting the same hash for M and M’ such that M != M’ -> undetected error

    - Assuming errors and hash are random

  - Error correction is more expensive, not worth it when retransmission is few ms

  - xxxxxxx -> xxxxxxxppp -> ppp -> needed addition to make xxxxxxxppp divisible by 7 -> receiver will ensure xxxxxxxppp is divisible by 7 -> extract xxxxxxx as data bits (we add 3 bits because we add up to 6 to make num divisible by 7 -> 110 is 3 bits.

  - CRC uses mod 2 arithmetic

    - Addition = subtraction = xor

      - 10001 + 11100 = 01101

    - Multiply

      - Shift + XOR

      - 10111 * 11 = 11001 (shifting just as normal mult -> use XOR when adding the terms)

    - Division

      - M = 1001, G = 101 => r = 3 (num bits of G)  [If first bit of G is 1, then the remainder will be r-1 bits)

      - M’ = 100100 (add r-1 = 2 bits to M)

      - 1011

      - _______

      - 101 | 100100

      - 101

      - 0011

      - 000

      - 110

      - 101

      - 0110

      - 101

      - 011

      - Remainder t = 11 (CRC).    [you no longer take G-t as CRC (only t)]

      - Final message = 100111 (which is mod 2 divisible by G=101

Each codeword has a set of valid codewords.

The idea is to maximize hamming distance between two adjacent valid codewords.

Parity has a hamming distance of 2

0 -> 000

1 -> 111

Hamming distance is 3

Hamming distance – 1 == resilience to error

Hamming distance of d:

C1 -------- d ---------- C2

|

|

|

d

|

|

|

C3

Any error of d – 1 bits on a valid codeword will result in an invalid codeword, so it will be dropped. => All d – 1 bit errors will be detected

Hamming distance of CRC is 4 (all 1-bit and 2-bit errors are detected… all odd bit errors are detected (3), but 4 bit errors are not detected)

Error correction: If hamming distance is 2d + 1, we can correct all d bit errors by going to the closest valid codeword. Error of > d bits will result in an incorrect correction

Media Access Control

- Reducing collisions

- Aloha

  - Fixed sized frames

  - Unslotted: within 2T will cause collision

    - Block size T. Any other frame starting before T seconds or during the T seconds (total 2T)

  - Slotted: within T will cause collision.

    - 0 … T … 2T … 3T

      - If frame is ready to be sent in (0, T] will be sent at T. Any frame ready to be sent (T, 2T] will be sent at 2T. => within the same T interval causes collision

- Ethernet

  - Variable sized frames

  - No acks in ethernet

  - Carrier sense

    - Don’t transmit/stop transmitting when you hear another signal

      - But if you already started sending data when a collision is detected, send more bits just to get enough collision signal so the other sender also detects the collision. (Jam)

  - No max packet size

  - There is a min packet size

    - If sender sends a long enough frame, then it will detect a collision

  - Sender needs to somehow know that a frame is dropped without acks

  - Ethernet has frames with a period of silence to determine end of frames

    - But we still need a length field to determine the padding bits that were used to reach 64 bytes (recall 64 bytes is required to ensure sender detects a collision of the frame it just sent)

- Statistical multiplexing

  - Bandwidth B, send B/x where x is number of busy users

  - Share

Questions

- Difference between Lan and Ethernet?

- Eye pattern? What’s the idea

- Binary exponential backoff??

- Doesn’t using hub completely chance way collisions happen we learned today?

Media Access Control
      Ethernet

- One big wire between a number of nodes

- Carrier sense possible: any node can detect collisions on the common wire

802.11

Hidden terminal problem

- A -> B -> C

- C cannot detect collision since it would happen on common terminal B

Solution: RTS/CTS

- Request to send / clear to send

- ( A ( ) B  ( ) C )

  - When B sends, it goes through everyone in his range (both A and C). But A and C cannot hear each other

- A sends RTS to B

- B sends CTS

- When C overhears a CTS sent to A, it keeps quiet for duration of the transfer

Ethernet for short distance

Interconnecting ethernets (data links) without routers

MAC address unique (48 bits)

IP address temp (32 bits)

Multicast

- Ethernet is broadcast medium -> can send to a set of people

- MAC msb:

  - 0 -> multicast broadcast

  - 1 -> 1 sending to 1 receiver

- MAC address on all devices are unique:

  - First three bytes are manufacturer (intel, dell, etc.)

- Hardware looks for a set of addresses: its own MAC address and the multicast addresses

  - If an address matches anyone in a set

- Hardware passes up to software via interrupt only when address matches the few addresses we are listening to

  - Software makes a call to hardware to specify MAC addresses in the set

- Hubs

  - N wires going into a hub and all connected together in series.

Bridges

- Ethernet 1 (I1) ----- bridge ------ Ethernet 2 (I2)

  - Bridge stores a mapping of MAC to I1 | I2

  - Algorithm:

    - Receiver F(D, S) on Interface X:

      - I = Loopup(D)

      - If X == I:

        - Drop

      - Else:

        - Send F to interface I

  - Adjust algo to learn the Loopup table based on source addresses

  - Flooding: Haven’t learned destination address yet, so we send it to all other interfaces

  - Filtering: We learned the interface of the destination, but source and destination are on the same interface, so we drop the packet

- All flooding stops soon due to acks

- How to handle when device changes interface (moves to another location)

  - Add timers that will reset each entry in the Lookup table

- Transparency

  - A bridge is transparent if you can remove the bridge and the nodes will not know the difference.. Router is not transparent because nodes need to directly communicate with the router

- Promiscuous Receive

  - Bridge must receive everything (read everything)

Realization

- Problem: ethernet runs at 10Mbs and routers run on Kbs, so we couldn’t efficiently use routers to connect ethernets. Also, there was no standard routing protocol

- Bridge needs to be fast

  - Bridge picks everything on the ethernet. Router only picks packets addressed to it

  - Solution:

    - Runs 10Mb/s

    - Memory shared by two ethernet chips connected to the ethernet interfaces

    - Binary search lookup engine

    - CPU talks to binary search lookup engine to perform the algorithm (flooding, filtering, etc.)

  - Bridges cannot have cylcles (topology must be a tree)

  - Every connection to the ethernet has a mac address, so bridges have at least 2 addresses

- Spanning tree

  - Pick root (bridge with minimum MAC), then build a minimum spanning tree

  - The distributed algo

    - All bridges send message (Root, Distance, Own ID)

    - Initially, all bridges think they are the root (at distance 0)

    - They broadcast their state and update their state accordingly on receive

    - Each ethernet picks designated bridge (db) that is closest to root

- Bridges vs. Routers

  - Both can connect ethernets

  - Bridges connect Ethernets within data link

  - Routers need a new layer (IP)

  - IP: 32 bits (written in 4 decimals separated by dots)

  - MAC: 48 bits, in pairs of hex

    - Everything connected to the ethernet has a MAC address

  - From bridge POV, routers are just devices with MAC connected to ethernet

  - From router POV, ethernets connected via bridges are just one big ethernet

- Routers

  - When you need a mac address on a line, routers broadcast ARP message on a line and ask everyone to respond with their MAC addresses. Then, that info is cached in an ARP table

  - Translate from IP to MAC using ARP

- Bridges are bad

  - Address Incompatibility and Max Packet Size incompability

    - Router has ability to chop router into pieces and give sequence number... Bridge has no choice but to drop packet if an intermediary ethernet does not support address or packet size

  - Routers are fine with handling prefixes of IP… Bridge addresses are flat (48 bit) without any hierarchy, so need to store entire addresse

  - Spanning tree is bad in wide area

    - Fine in local area network, but in wide area, you might need to travel a long distance even if destination is close if the bridge happened to be off

- Levels of addressing

  - Human friendly: Bruinlearn.ucla.edu    (e.g., “file.txt”)

  - Locator: IP address			   (e.g., file_descriptor)

  - HW level: MAC address		   (e.g., plate, sector)

- Translation from Human friendly to locator: DNS server

- Translation from IP to MAC: ARP